WWF-Canada Carbon Training

Grassland Carbon Measurement — In-Field Skill Checklist

Participant:  ________________________________     Date:  _____________     Site:  _____________

Work through each skill with your trainer. Check the left ☐ when you have practised it; check the right ☐ when your trainer confirms you can do it unsupervised. Section 5 — root separation — is the one most people arrive without.

|  | Skill | Description / Key Actions | Complete ✔ |
| --- | --- | --- | --- |
| SECTION 1 — Sampling Design |  |  |  |
| Study area and strata |  |  |  |
| ☐ | Define the study area | Draw the boundary and write down the RULE used. Exclude wetland inclusions, rock outcrop, roads and shelterbelts — in parkland and interior BC these can be a substantial fraction of a quarter section. | ☐ |
| ☐ | Stratify on MANAGEMENT first | Grazing regime, cultivation history, years since fire, native vs seeded. Not vegetation cover — management is the dominant variable in a grassland and it is mappable from a fence line. | ☐ |
| ☐ | Get the management history | From the landholder. Grazing and stocking, burn years, when cultivation stopped, seeding. This conversation is part of the method, not a courtesy. | ☐ |
| How many cores |  |  |  |
| ☐ | Size SOIL and ROOTS separately | Roots are 4–5x more variable than soil carbon, so at the same target they need 4–5x the cores. Set a looser target for roots (±40% rather than ±20%) or wash a random subset. | ☐ |
| ☐ | Apply the t-floor | Cochran's n uses z, which assumes a known SD. You estimate it, so add the two extra cores the t-multiplier needs. | ☐ |
| ☐ | Decide if these are permanent plots | A campaign designed for a one-off stock is often unusable as a monitoring baseline. | ☐ |
| SECTION 2 — Plot Setup |  |  |  |
| Laying out |  |  |  |
| ☐ | Lay out the plots | Small 0.25 m² · medium 16–100 m² · large 400 m² if trees are in scope. Soil core offset from the quadrat. | ☐ |
| ☐ | Record coordinates and GNSS accuracy | At the plot centre. | ☐ |
| ☐ | Mark a permanent plot so it can be refound | Grassland has nothing to tag. Driven rod with recorded depth, buried magnet, AND a bearing and distance from something durable. | ☐ |
| ☐ | Complete the plot log BEFORE touching anything | Grassland type, management, grazing regime, years since fire, native or seeded, canopy cover. | ☐ |
| Photographs |  |  |  |
| ☐ | 14-photo series | Straight down, straight up, and three per cardinal direction. | ☐ |
| ☐ | Quadrat before AND after clipping | From the same position. The only record of what the standing crop looked like. | ☐ |
| SECTION 3 — Vegetation (before any coring) |  |  |  |
| Order of work |  |  |  |
| ☐ | Finish ALL vegetation work before coring | Coring is destructive. A corer hole in a quadrat you have not yet clipped is a lost plot. | ☐ |
| Clip and weigh |  |  |  |
| ☐ | Clip at GROUND LEVEL | Not grazing height — that is not a defined datum and will not repeat between crews or visits. | ☐ |
| ☐ | Separate live from dead | Standing dead and litter hold carbon but are not production. Merging them later is easy; splitting them is not. | ☐ |
| ☐ | Confirm peak growing season, and record the date | Peak is the only phenological point that repeats between sites and years. A clip-and-weigh without a date cannot be interpreted. | ☐ |
| Shrubs and trees |  |  |  |
| ☐ | Measure shrubs non-destructively | Crown L × W × H, or stem diameter at 0.3 m. Check the species is in the coefficient table before relying on an allometric. | ☐ |
| ☐ | Apply the tree-cover threshold consistently | Default ≥25% canopy. Savannah sits right on this boundary, so the rule matters more here than anywhere else. | ☐ |
| ☐ | Note whether trees are open-grown | Savannah oaks have short boles and wide crowns; forest-fitted equations extrapolate poorly to them. | ☐ |
| SECTION 4 — Soil Coring |  |  |  |
| Taking the core |  |  |  |
| ☐ | Measure and record the corer diameter | It sets sample volume and therefore every bulk density. Out by 10% puts every carbon stock out by 21%. Write it on the corer. | ☐ |
| ☐ | Core offset from the quadrat | And record the offset, so the next visit can avoid this visit's holes. | ☐ |
| ☐ | Do not core wet or frozen ground | Both wreck bulk density. | ☐ |
| ☐ | Record depth driven AND length recovered | If they differ, the increments are not where you think. Do NOT stretch them back out. | ☐ |
| Sectioning |  |  |  |
| ☐ | Section at planned increments, record ACTUAL depths | A truncated increment scaled as a full one is a silent error. | ☐ |
| ☐ | Record depth to refusal and what stopped you | Bedrock, cemented layer, stone. This is data, not a failure. | ☐ |
| ☐ | Estimate coarse fragments per increment | Rocks hold no carbon, and neither WWF guide's equations correct for them. Mandatory in interior BC. | ☐ |
| ☐ | Ask the lab which bulk-density basis they report | Fine earth over total volume, or over fine-earth volume. Getting it wrong understates a stony site by up to a third — or double-corrects it. | ☐ |
| ☐ | Label bags with plot, core and DEPTH INTERVAL | Not just an increment number. | ☐ |
| SECTION 5 — Root Separation |  |  |  |
| The decision that comes first |  |  |  |
| ☐ | Decide the root/soil boundary BEFORE the lab | Sieve roots out and analyse root-free soil, OR leave fine roots in the soil pool and report only coarse roots separately. Both are defensible. Silence is not, and silence is the default if nobody decides. | ☐ |
| ☐ | Record the sieve mesh | Roots finer than the mesh are lost, so fine-root biomass is a known underestimate. Without the mesh size the figure cannot be interpreted. | ☐ |
| Processing |  |  |  |
| ☐ | Weigh the field-moist increment first | Before anything is removed. | ☐ |
| ☐ | ▶ TEAM Soak and disperse, then wash over nested sieves | 2 mm for the fine/coarse split, plus a finer mesh to catch fine roots. | ☐ |
| ☐ | Separate live from dead | Flotation, then by eye — live roots are paler, turgid, elastic, with intact cortex. | ☐ |
| ☐ | Sort by diameter class | ≤2 mm fine, >2 mm coarse. A convention a crew can apply consistently, not a biological boundary. | ☐ |
| ☐ | Dry at 60–70 °C — NOT 105 °C | 105 °C is the soil bulk-density convention and can volatilise organics in root tissue. Two samples, two ovens, two conventions. | ☐ |
| ☐ | Ash-correct the root mass | Washed roots retain adhering mineral soil. Combust a subsample and subtract the residue, or root biomass is systematically too high. | ☐ |
| ☐ | Retain the root-free soil for carbon analysis | Under the sieve-first option, this is what goes to the lab. | ☐ |
| SECTION 6 — Calculations and Reporting |  |  |  |
| Calculations |  |  |  |
| ☐ | Soil carbon per increment | Bulk density × (carbon % ÷ 100) × thickness, to kg C/m². Apply the coarse-fragment correction ONCE. | ☐ |
| ☐ | Root carbon per increment | Ash-corrected dry mass ÷ core area, scaled to m², × carbon fraction. | ☐ |
| ☐ | Report soil to 30 cm AND to the full depth cored | 30 cm is the comparability depth. It is a floor, not the total. | ☐ |
| ☐ | Check achieved precision, per pool | Soil and roots against their separate targets. Use t, not z, at small n. | ☐ |
| Reporting |  |  |  |
| ☐ | Report the standing crop separately | Never merged into the soil figure. | ☐ |
| ☐ | Flag a root total that stops at the bottom of the core | It is a MINIMUM. Native grassland roots reach metres. | ☐ |
| ☐ | State what is excluded | Shrub roots, litter, and below-ground tree biomass are in no pool this workshop measures. Name the gaps. | ☐ |

Trainer:  ________________________________     Signature:  ________________________________     Date:  _____________

Sources: Measuring Carbon in Vegetation (Non-Tree) and Measuring Carbon in Non-Peat Soils (WWF-Canada). Sampling-design items follow Part 2 of the Grassland Carbon Workshop; root separation follows Part 3A.