WWF-Canada Carbon Training

Grassland Field Data Sheet

Field sheet for grassland carbon — soil, ROOTS and vegetation. Field names match the Grassland Carbon Calculator tabs, so transcription is a straight copy. Lab columns (bulk density, organic carbon, root mass, ash fraction) are filled when results come back and are not on this sheet.

Project:  ____________________________________     Recorder:  ________________________     Page ____ of ____

| 1. PLOT & SITE LOG — one block per plot; fill this FIRST |  |  |
| --- | --- | --- |
| Plot ID: | Site ID: | Study area: |
| Location: | Date: | Elevation (m): |
| Latitude: | Longitude: | GNSS accuracy (m): |
| Datum: | Canopy cover (%): | Slope position: |
| Management history — from the landholder, not from the site. These are the stratification variables. |  |  |
| Grassland type (prairie / aspen parkland / Black Oak savannah / interior BC bunchgrass / other): ______________________ Management (never cultivated / cultivated and reseeded / long-term tame pasture / hayed / unknown): ______________ Grazing regime (none / season-long / rotational / heavy / unknown): ________________________________________ Years since fire: ____________ Native or seeded: ________________________________________________ |  |  |

2.  SOIL DATA — one row per depth increment

Core ID:  ____________     Corer internal diameter:  ______ cm     Bulk density basis the lab reports:  whole sample ☐   fine fraction ☐   not confirmed ☐

Record ACTUAL top and bottom depths, not the ones you planned — a truncated increment scaled as a full one is a silent error. If you hit refusal, record the depth and what stopped you: that is data, not a failure.

| Inc. # | Top depth (cm) | Bottom depth (cm) | Depth driven (cm) | Length recovered (cm) | Coarse fragments (% vol) | Notes — horizon change, refusal, moisture |
| --- | --- | --- | --- | --- | --- | --- |

3.  ROOT BIOMASS — one row per increment per diameter class

Core ID:  ____________     Finest sieve mesh used:  ______ mm     Roots sieved out BEFORE soil carbon analysis?   YES ☐    NO ☐

Roots are MEASURED here, not estimated from a root:shoot ratio. Oven-dry mass is taken at 60–70 °C — NOT the 105 °C used for soil bulk density. Mass and ash fraction come back from the lab; this sheet records what you sampled.

| Top depth (cm) | Bottom depth (cm) | Diameter class (≤2 mm / >2 mm) | Live or dead | Notes |
| --- | --- | --- | --- | --- |

Roots still present in the deepest increment?   YES ☐    NO ☐

If YES, the root total is a MINIMUM — the profile did not end, your coring did. Native grassland roots reach metres.

4.  VEGETATION DATA — one block per plot

Sampling date:  ____________     Peak growing season?   YES ☐    NO ☐    UNKNOWN ☐

A standing crop is NOT a stock — it grows from nothing each spring and is gone by autumn. Clip at GROUND LEVEL, not grazing height. Photograph the quadrat before and after.

| Small plot — clip and weigh |  |
| --- | --- |
| Small plot area (m²): 0.25 ☐ other: ______ | Quadrat photographed before and after? Y ☐ N ☐ |
| Clip fresh mass — LIVE (g): | Clip fresh mass — DEAD (g): |
| Bags labelled: ☐ | Number of bags: |
| Medium plot — shrubs (measure and predict) |  |
| Medium plot area (m²): | Number of shrubs measured: |
| Species recorded: ☐ | Crown L × W × H or stem diam. at 0.3 m recorded: ☐ |
| Large plot — trees. Only if canopy cover ≥ the threshold set in the calculator (default 25%). Use the FORESTS tree datasheet. |  |

Canopy cover (%):  ________        Trees measured?   YES ☐    NO ☐    N/A ☐        Trees open-grown?   YES ☐    NO ☐

Open-grown savannah trees have short boles and wide crowns. Allometric equations fitted on forest-grown stems extrapolate poorly to them — record the form so the biomass figure can be reported as more uncertain.

Sources: Measuring Carbon in Vegetation (Non-Tree) and Measuring Carbon in Non-Peat Soils (WWF-Canada). Root separation is not covered by either guide; see the Grassland workshop Part 3A.